Design Document

cpu_emu

(In Development)

Joseph MacDonald

Independent Project

# Contents

Table of Contents

Contents	2

Section 1 - ISA Design	3

1.	Instruction and Data Design	3

a.	Notes	3

b.	Instruction Formats	4

Section 2 – Emulated System Design	5

1.	Finite State Machine Design	5

2.	Data Handling Design	7

Section 3 – Emulator Design	8

# Section 1 - ISA Design

## Instruction and Data Design

### Notes

Instr/Data Bit of 1 indicates presence of an instruction

Reg/Literal Bit of 1 indicates the presence of a register reference

All instructions and data are stored in 64-bit chunks

Memory is referenced using 32-bit addresses

All 64-bit data values are stored in Two’s Complement form

ASCII-bit indicates whether data should be output in an ASCII form

### Instruction Types

| Type | Format | Description | Examples |
| --- | --- | --- | --- |
| ADD | Type 1 | Performs addition on two input values, stores in specified register. Can take either register or literal operands | ADD R0, R1, R2 ADD R1, R1, #10 ADD R10, #15, #23 |
| SUB |  | Subtracts second value from first, stores in specified register. Can take either register or literal values as operands | SUB R0, R5, R4 SUB R0, #100, R7 SUB R9, R6, #10 |
| MULT |  | Multiplies two values together, stores in specified register. Can take register or literal operands. In effect, repeat addition for specified amount | MULT R7, R4, R2 MULT R4, #42, R9 MULT R4, R6, #9 |
| AND |  | Performs a bitwise AND operation on the two input values. Stores in specified register, can take register or literal operands | AND R0, R2, R9 AND R3, #12, R9 AND R5, #12, #34 |
| OR |  | Performs a bitwise OR operation on the two input values. Stores in specified register, can take register or literal operands | OR R3, R3, R4 OR R5, #92, #100 OR R7, #12, R1 |
| XOR |  | Performs a bitwise XOR operation on the two input values. Stores in specified register, can take register or literal operands | OR R6, R11, R2 OR R12, #99, R1 OR R7, #104, R4 |
| NOT | Type 2 | Performs a bitwise NOT operation on the input. Stores in specified register, can take a register or literal operand | NOT R1, R1 NOT R10, #2 |
| FLIP |  | Flips the sign of the input, be that literal or register | FLIP R4, R8 |
| STR | Type 3 | Stores specified register contents at the memory address supplied | STR R2, &1223ABCD STR R0, &A1B2C3D4 |
| LDR |  | Loads the value stored at the specified memory address into the register supplied | LDR R5, &AEEC1442 LDR R8, &12345678 |
| CMP | Type 4 | Compares two input values, setting ALU flags but not changing the value in any register. Can take literal or register inputs | CMP R4, R7 CMP #9, R2 CMP R10, #0 |
| B | Type 5 | Performs a branch operation, setting the PC to the position specified address if any conditions are met. Find branch conditions in table ‘c. Branch Conditions’ | B &ABCD1234 BNE &12344321 BLT &FFECFDDA |
| HLT | Type 6 | Stops the clock, ending any running operations | HLT |
| OUT | Type 7 | Outputs the value in the specified register to an external file. Will output in ASCII form if the ‘A’ flag is set | OUT R R7 OUT A R1 |

### Instruction Formats

#### Type 1 – Arithmetic & 2-Operand Bitwise Operations

[Instr Type]{4} [Return Register]{4} [Reg/Literal Bit 1] [Reg Reference/Literal Value]{16} [Reg/Literal Bit 2] [Reg Reference/Literal Value]{16} [Padding]{20}

#### Type 2 – Single-Operand Bitwise Operations

[Instr Type]{4} [Return Register]{4} [Reg/Literal Bit 1] [Reg Reference/Literal Value]{16} [Padding]{40}

#### Type 3 – Memory Operations

[Instr Type]{4} [Referenced Register]{4} [Memory Address Reference]{32} [Padding]{24}

#### Type 4 – Comparator Operations

[Instr Type]{4} [Reg/Literal Bit 1] [Reg Reference/Literal Value]{16} [Reg/Literal Bit 2] [Reg Reference/Literal Value]{16} [Padding]{24}

#### Type 5 – Branch Operations

[Instr Type]{4} [Branch Condition]{4} [Memory Address Reference]{32} [Padding]{24}

#### Type 6 – Control Operations

[Instr Type]{4} [Padding]{60}

#### Type 7 – Output Operations

[Instr Type]{4} [Target Register]{4} [ASCII Bit] [Padding]{54}

# Section 2 – Emulated System Design

## Finite State Machine Design

| State | No. | Description/Steps | Next State(s) |
| --- | --- | --- | --- |
| Fetch | 0 | Retrieve next instruction from memory Call cache - If cache hit – Decode Instruction - If cache miss – Stall, wait for RAM | Decode Stall |
| Decode | 1 | Decodes 64-bit instruction from Fetch/Stall Parses into parsedInstruction struct | Execute |
| Execute | 2 | Execute instruction from Decode In most cases, move to Fetch In case of LDR, move to Stall until the data returns from RAM | Fetch Stall |
| Stall | 3 | Wait for either instruction fetch or LDR instruction to return value from RAM Store returned instruction in MemoryInstructionRegister, ready for decoding Move to MemoryComp if data returned, store in Memory Data Register | Decode MemoryComp |
| MemoryComp | 4 | Takes data stored in Memory Data Register, stores in the appropriate register in the CPU | Fetch |

## Data Handling Design

To truly emulate the CPU/Cache/RAM interaction, a new CPU component, the DataAccessManager, is used as a middleman to manage components and place data and addresses in the appropriate positions. The CPU itself only ever interacts with the DAM. This does mean that, in a slight break from reality, the L1 Cache is not built into the CPU object itself, but this was thought to be necessary to keep the structure as simple, readable and efficient as possible. The DAM handles two levels of cache, L1 and L2, each of different sizes, and with L1 having priority. The DAM organises dataflow between the caches, the CPU, and the memory buses, and allows for cascading data to flow from L1 into L2 in the event of an overwrite in L1.

The DAM also manages memory interaction.

Both cache modules are built to a LRU format, using a queue to keep the most relevant data.

IPC Data Protocol – WORK-IN-PROGRESS

#### Dashboard to Emulator Messages

| Message | String Format |
| --- | --- |
| Get Memory Data from Location | GET//<Memory Address> |
| Increase Clock Speed | INC_CLK |
| Decrease Clock Speed | DEC_CLK |
| Set Clock Speed | SET_CLK//<Val> |

#### Emulator to Dashboard Messages

| Message Type | String Format |
| --- | --- |
| Update Register Value | REG//<Reg Num>//<Val> |
| Update State Value | STATE//<State No.> |
| Update Instruction Values | INSTR//<Current Instr Hex>//<Incremented Instr Hex> |
| Return Memory Data | RAM//<Memory Address>//<Memory Data> |
| L1 Cache Utilisation | L1_UTIL//<Val> |
| L2 Cache Utilisation | L2_UTIL//<Val> |